FPGA 32-Bit CPU Documentation

1. Overview

This document describes a custom FPGA 32-bit CPU architecture featuring a simple 2-stage pipeline, decimal immediates, and 32-bit word addressing (4 bytes per address).

2. Registers

Registers are named A through AE (A = R1, B = R2, etc.). R0 is a constant zero register and ignores writes.

| Register | Description |
| --- | --- |
| R0 | Constant 0 (read-only) |
| A (R1) | General purpose |
| B (R2) | General purpose |
| C (R3) | General purpose |
| D (R4) | General purpose |
| E (R5) | General purpose |

FLAGS register stores: N (Negative), E (Equal), G (Greater)
Program Counter (PC) increments by 1 unless modified by a branch or jump.

3. Two-Stage Pipeline

| Cycle | Operation |
| --- | --- |
| N | Fetch: Send address to BRAM, set read bit, increment PC |
| N+1 | Execute: Read data from BRAM, execute instruction, send next address |

READ and WRITE require 2 cycles and insert a NOP bubble.

| Instruction | Pipeline Behavior |
| --- | --- |
| READ | N: Prepare address B+imm, stall PC N+1: Read memory, write result to destination, resume pipeline |
| WRITE | N: Prepare address A+imm, stall PC N+1: Perform write, insert NOP, resume pipeline |

4. Instruction Formats

| Type | Bit Layout (MSB → LSB) |
| --- | --- |
| R-Type | opcode[31–26] ¦ rd[25–21] ¦ rt[20–16] ¦ rs[15–11] ¦ sh[10–6] ¦ func[5–0] ¦ ? |
| I-Type | opcode[31–26] ¦ rd[25–21] ¦ rs[20–16] ¦ imm[15–0] |
| J-Type | opcode[31–26] ¦ imm[25–0] |

5. R-Type Instructions

| Mnemonic | Func | Description |
| --- | --- | --- |
| ADD | 00000 | d = s + t |
| SUB | 00010 | d = t - s |
| AND | 00100 | d = t AND s |
| OR | 00101 | d = t OR s |
| XOR | 00111 | d = t XOR s |
| SLL | 01000 | Shift left logical by sh |
| SRL | 01001 | Shift right logical by sh |
| SRA | 01010 | Shift right arithmetic by sh |
| RCL | 01011 | Rotate left by sh |
| RCR | 01100 | Rotate right by sh |
| CMP | 01110 | Compare t - s (set flags) |

6. I-Type Instructions

| Mnemonic | Opcode | Operation |
| --- | --- | --- |
| ADDI | 000010 | d = t + imm |
| SUBI | 000100 | d = t - imm |
| ANDI | 000110 | d = t AND imm |
| ORI | 000111 | d = t OR imm |
| XORI | 001001 | d = t XOR imm |
| CMPI | 001010 | Compare t - imm, set flags |
| MOV | 001000 | d = t |
| LDI | 001100 | d = imm |

7. J-Type (Control Flow)

| Mnemonic | Opcode | Description |
| --- | --- | --- |
| JMP | 010101 | Jump absolute |
| JMPN | 010110 | Jump if Negative |
| JMPE | 010111 | Jump if Equal |
| JMPG | 011000 | Jump if Greater |
| BAL | 011001 | Branch always (relative) |
| BRE | 011010 | Branch if Equal |
| BRN | 011011 | Branch if Negative |
| BRG | 011100 | Branch if Greater |

8. Example Program

LDI A, 1000   ; Load address 1000 (decimal)
READ B, A, 0  ; Read from MEM[A+0] into B
ADDI B, B, 1  ; Increment B by 1
WRITE A, B, 0 ; Store B back to MEM[A+0]
BAL -5      ; branch to begining of the code

Fibonacci:

fibonacci: LDI A 0

LDI B 1

IN 1 E

DEC E E 1

DEC E E 1

fib_loop: BRN fib_end

ADD C A B

MOV A B

MOV B C

DEC E E 1

BAL fib_loop

fib_end: OUT 1 C

BAL choice

Hello World!:

LDI A 10 ;Pointer to the begining of the string

loop: READ B A 0

CMPI _ B 0

BRE end

OUT 3 B

ADDI A A 1

BAL loop

end: NOP

RGB:

rgb: LDI A 0

LDI B 0

rgb_loop: ANDI C B 127

ADD C C C

MOV A C

ANDI C B 16256

SLL C C 10

ADD A A C

OUT 2 A

OUT 2 B

ADDI B B 1

BAL rgb_loop

9. Assembler and Toolchain

Purpose

The included C++ assembler (assembler.cpp) translates human-readable assembly code into 32-bit machine words for the FPGA CPU. It supports labels, immediates (decimal or hexadecimal with '0x' prefix), and symbolic register names. Output is formatted as 'v3.0 hex words addressed', suitable for loading into FPGA block RAM initialization files.

Workflow

1. Run the assembler in a terminal:
   ./assembler

2. Enter assembly lines interactively. Type '/done' to finish and compile.
3. The assembler prints each instruction in binary and hexadecimal and generates a final block-RAM image containing eight 32-bit words per line.

Label Resolution

Labels followed by a colon (:) mark instruction addresses. Branch and jump immediates are automatically replaced with relative offsets computed as (target - current - 1).

Register Naming

Registers can be referenced symbolically or numerically:
- R0 or '_' = constant zero register
- A → R1, B → R2, ... AE → R31
- R1–R31 can also be used directly.

Immediate Syntax

Immediates may be specified in either format:
- Decimal: e.g., 42 or -5
- Hexadecimal: e.g., 0xFF
Signed 16-bit and 26-bit fields are automatically masked.

Instruction Encoding

Each instruction is encoded according to its type:
- R-Type: opcode 000001 + 5-bit function code
- I-Type: opcode defined by operation (see sections 5–6)
- J-Type: branch/jump with 26-bit offset

Field layout:
[31–26] opcode | [25–21] rd | [20–16] rt | [15–11] rs | [10–6] sh | [5–1] func | [0] flag

Special Features

- Comments start with ';'
- Duplicate labels produce warnings
- 'NOP' generates a zero instruction
- 'READ' and 'WRITE' automatically insert required pipeline stalls
- Output is padded to 8-word boundaries for memory alignment

Required Knowledge

To modify or extend the assembler, you should understand:
- Bit-field packing of instructions
- Register file conventions (R0 ignores writes)
- Branch offset calculation and program counter behavior
- Core C++ concepts: streams, maps, and bitwise operations

Example Session

> LDI A 10
> ADDI B A 1
> OUT 3 B
> /done

Assembler output:
LDI   -> 00110000001000000000000000001010  0C20000A
ADDI  -> 00001000010000100000000000000001  08420001
OUT   -> 01001000000000110000000000000011  12030003

v3.0 hex words addressed
00: 0c20000a 08420001 12030003 00000000 00000000 00000000 00000000 00000000